library IEEE; use IEEE.STD_LOGIC_1164.ALL;

entity gate_4 is

port(w, x, y, z: in std_logic; f: out std_logic);

end gate_4;

-- nor4

architecture nor4_basic of gate_4 is

component nor_2 is

port(x, y: in std_logic; f: out std_logic);

end component nor_2;

signal wire0, wire1, wire2, wire3: std_logic;

begin

nor2_0: nor_2 port map(w, x, wire0);

nor2_1: nor_2 port map(wire0, wire0, wire1);

nor2_3: nor_2 port map(y, z, wire2);

nor2_4: nor_2 port map(wire2, wire2, wire3);

nor2_5: nor_2 port map(wire1, wire3, f);

end nor4_basic;

architecture nor4_primitive of gate_4 is

component or_2 is

port(x, y: in std_logic; f: out std_logic);

end component or_2;

component not_1 is

port(x: in std_logic; f: out std_logic);

end component not_1;

signal wire0, wire1, wire2: std_logic;

begin

or2_0: or_2 port map(w, x, wire0);

or2_1: or_2 port map(y, z, wire1);

or2_3: or_2 port map(wire0, wire1, wire2);

not1_0: not_1 port map(wire2, f);

end nor4_primitive;

architecture nor4_behavior of gate_4 is

begin

f <= '0' when (w = '1' or x = '1' or y = '1' or z = '1')

else '1';

end nor4_behavior;

architecture nor4_equation of gate_4 is

begin

f <= NOT (w OR x OR y OR z);

end nor4_equation;

-- or4

architecture or4_basic of gate_4 is

component or_2 is

port(x, y: in std_logic; f: out std_logic);

end component or_2;

signal wire0, wire1: std_logic;

begin

or2_0: or_2 port map(w, x, wire0);

or2_1: or_2 port map(y, z, wire1);

or2_3: or_2 port map(wire0, wire1, f);

end or4_basic;

architecture or4_primitive of gate_4 is

component or_2 is

port(x, y: in std_logic; f: out std_logic);

end component or_2;

signal wire0, wire1: std_logic;

begin

or2_0: or_2 port map(w, x, wire0);

or2_1: or_2 port map(y, z, wire1);

or2_3: or_2 port map(wire0, wire1, f);

end or4_primitive;

architecture or4_behavior of gate_4 is

begin

f <= '1' when (w = '1' OR x = '1' OR y = '1' OR z = '1')

else '0';

end or4_behavior;

architecture or4_equation of gate_4 is

begin

f <= (w OR x OR y OR z);

end or4_equation;

-- xor

architecture xor4_basic of gate_4 is

component xor_2 is

port(x, y: in std_logic; f: out std_logic);

end component xor_2;

signal wire0, wire1: std_logic;

begin

xor_0: xor_2 port map(w, x, wire0);

xor_1: xor_2 port map(y, z, wire1);

xor_3: xor_2 port map(wire0, wire1, f);

end xor4_basic;

architecture xor4_primitive of gate_4 is

component and_2 is

port(x, y: in std_logic; f: out std_logic);

end component and_2;

component not_1 is

port(x: in std_logic; f: out std_logic);

end component not_1;

signal wire00, wire01, wire02, wire03, wire04, wire05, wire06, wire07: std_logic;

signal wire10, wire11, wire12, wire13, wire14, wire15, wire16, wire17: std_logic;

signal wire20, wire21, wire22, wire23, wire24, wire25, wire26: std_logic;

begin

-- W XOR X

and02_0: and_2 port map(w, x, wire00);

not01_0: not_1 port map(wire00, wire01); -- W NAND X

and02_1: and_2 port map(w, wire01, wire02); -- W AND (W NAND X)

and02_2: and_2 port map(x, wire01, wire03); -- X AND (W NAND X)

not01_1: not_1 port map(wire02, wire04); -- W NAND (W NAND X)

not01_2: not_1 port map(wire03, wire05); -- X NAND (W NAND X)

and02_3: and_2 port map(wire04, wire05, wire06); -- (W NAND (W NAND X)) AND (X NAND (W NAND X))

not01_3: not_1 port map(wire06, wire07); -- wire07 = W XOR X

-- W XOR X XOR Y (wire07 XOR Y)

and12_0: and_2 port map(wire07, y, wire10);

not11_0: not_1 port map(wire10, wire11);

and12_1: and_2 port map(wire07, wire11, wire12);

and12_2: and_2 port map(y, wire11, wire13);

not11_1: not_1 port map(wire12, wire14);

not11_2: not_1 port map(wire13, wire15);

and12_3: and_2 port map(wire14, wire15, wire16);

not11_3: not_1 port map(wire16, wire17); -- W XOR X XOR Y

-- W XOR X XOR Y XOR Z (wire17 XOR Z)

and22_0: and_2 port map(wire17, z, wire20);

not21_0: not_1 port map(wire20, wire21);

and22_1: and_2 port map(wire17, wire21, wire22);

and22_2: and_2 port map(z, wire21, wire23);

not21_1: not_1 port map(wire22, wire24);

not21_2: not_1 port map(wire23, wire25);

and22_3: and_2 port map(wire24, wire25, wire26);

not21_3: not_1 port map(wire26, f); -- W XOR X XOR Y XOR Z

end xor4_primitive;

architecture xor4_behavior of gate_4 is

begin

f <= '1' when (w = '1' XOR x = '1' XOR y = '1' XOR z = '1')

else '0';

end xor4_behavior;

architecture xor4_equation of gate_4 is

begin

f <= (w XOR x XOR y XOR z);

end xor4_equation;

Figure 1.  VHDL for various 4 input gates

Figure 2.  nor4 basic

Figure 3.  nor4 primitive

Figure 4.  nor4 behavior

Figure 5.  nor4 equation

Figure 6.  or4 basic

Figure 7.  or4 primitive

Figure 8.  or4 behavior

Figure 9.  or4 equation

Figure 10.  xor basic

Figure 11.  xor primitive

Figure 12.  xor behavior

Figure 13.  xor equation

Figure 14.  xor equation RTL

Figure 15.  xor behavior RTL

Figure 16.  xor primitive RTL

Figure 17.  xor basic RTL

Figure 18.  or equation RTL

Figure 19.  or behavior RTL

Figure 20.  or primitive RTL

Figure 21.  or basic RTL

Figure 22.  nor equation RTL

Figure 23.  nor behavior RTL

Figure 24.  nor primitive RTL

Figure 25.  nor basic RTL